Online resources for solar & renewable  energy  systems sizing

- To assess local solar resource, use RETscreen :  http://www.retscreen.net/ang/home.php

  - download RETscreen 4 software  (or latest version), which is an Excel based tool

  - install and run software

  - (Might need to enable macros before running RETscreen.)

  - At top of page, select relevant “Project type”, technology, and off-grid choices

  - At bottom of page, Click on “Select climate data location”  and check the box “Show Data”

  - Explore: choice of location and see amount of solar energy average per month on flat surface

  - For orientation, tilt, etc.  open the tab “Energy model”, scroll down to electrical production system, and in “resource assessment” fields, enter tilt (from horizontal) and azimuth (= orientation, with 0 = south)

  - Example: what’s the average amount of solar energy in kWh/m²  in Portland in July on a solar panel facing south and tilted 25 degrees from the horizontal?

- For  pipe sizing and wire sizing online, use http://www.sunpumps.com/engineering/engineering.php

  - Click on the tab you need,

  - Fill in data or use down arrows

  - For pipe sizing, you need to use English units.

    - Useful conversion factors:

    - 1 gal = 3.78liter =>  1gpm = 3.78 l/min = 0.063 l/s

    - 1m³ = 1000 l

    - 10 m of H2O ~ 1 bar = 14 psi

    - 1 psi = 0.07bar

    - 1 in =2.54cm  (but a 1” diameter pipe actually has an inside diameter greater than 1 inch)

    - 1 ft = 0.304 m

  - Friction losses are usually expressed in equivalent length of pipe; 1ft/100ft  = 1m/100m or 1% of the pipe length

| Example 1: what’s the pressure drop in % in a pipe used to carry water with a flow rate of 10gpm, if the pipe is 1” nominal diameter, made of PVC and can withstand 160 psi of pressure? Example 2: using the same scenario, what diameter pipe do we need to use in order not to increase the friction loss by more than 5m over a length of 1km? |  |
| --- | --- |

  - Voltage drop in wires can be expressed in % over a given cable length   and for a same current it also correspond to a power loss in %

    - To use this tool, you must first figure out the current going through your wires.

Remember  P = V * I    therefore I = P/V

| Example 1: what’s the voltage drop in % in a 100W DC system operating at a battery voltage of 12V, if you use 10ft of AWG cables (common size for AC in houses)? Could you use this cable for a 1kW 12V system ? |  |
| --- | --- |