# METADATA

# Map name example:CA_bishop

The first two letters of the name are the two letter state abbreviations (CA = california), after the underscore, the 1:100,000 dlg name follows. Details on the file structure contained in file structure.doc. All applicable 1:24,000 DLGs, DEMs and 1:100,000 DLGs are contained in the map coverage.

Created by Patrick Bardel 8/99

# Data quality

The method to create this coverage is outlined in the document titled “instructions.doc.”  All data was obtained from USGS in SDTS format. The DEMs are all 1:24,000 scale. The DLGs are mostly 1:100,000 scale except where noted some are 1:24,000, refer to “7.5 DLGs.doc” for a summary of DLG scales. Maps with 7.5 DLG coverages are titled “coverage name_7.5_dlg.”

# Distribution information

This dataset can be obtained from Andrew Fountain at Portland State University. It is available in zip disk and FTP download. There is no fee for this data.

# Coordinate system description

Each 7.5-minute unit of DEM coverage (based on the 7.5-minute quadrangle) consists of a regular array of elevations referenced horizontally in the UTM projection coordinate system. Elevation units are in meters or feet relative to National Geodetic Vertical Datum of 1929 (NGVD 29) in the continental U.S. More details are available in the USGS dem metadata document.

Digital line graph (DLG) data are digital representations of cartographic information. DLG's of map features are converted to digital form from maps and related sources. Intermediate-scale DLG data are derived from USGS 1:100,000-scale 30- by 60-minute quadrangle maps. More details available in the document DLG metadata details.