National Spatial Data Infrastructure

USGS Geospatial Data Clearinghouse National Mapping and Remotely Sensed Data

1:100,000-scale Digital Line Graph (DLG)

Digital line graph (DLG) data are digital representations of cartographic information. DLG's of map features are converted to digital form from maps and related sources. Intermediate-scale DLG data are derived from USGS 1:100,000-scale 30- by 60-minute quadrangle maps. Intermediate-scale DLG's are sold in five categories: (1) Public Land Survey System; (2) boundaries (3) transportation; (4) hydrography; and (5) hypsography. All DLG data distributed by the USGS are DLG - Level 3 (DLG-3), which means the data contain a full range of attribute codes, have full topological structuring, and have passed certain quality-control checks. This is a data-set level implementation of the Federal Geographic Data Committee's Content Standards for Digital Geospatial Metadata.

Information about individual 100,000-scale DLG records can be obtained from the EROS Data Center.

Table of Contents

Identification Information (includes abstract)
Data Quality Information
Spatial Data Organization Information
Spatial Reference Information
Entity and Attribute Information
Distribution Information (includes ordering information)
Metadata Reference Information

Identification Information

Identification_Information:

Citation:

Citation_Information:

Originator:

U.S. Geological Survey or another mapping agency in

cooperation with USGS.

Publication_Date: 19850101

Title: 1:100,000-scale Digital Line Graphs

Publication_Information:

Publication_Place: Reston, Virginia

Publisher: U.S. Geological Survey

Description:

Abstract:

Digital line graph (DLG) data are digital representations of

cartographic information. DLG's of map features are

converted to digital form from maps and related sources.

Intermediate-scale DLG data are derived from USGS

1:100,000-scale 30- by 60-minute quadrangle maps. If these

maps are not available, Bureau of Land Management

planimetric maps at a scale of 1: 100,000 are used.

Intermediate-scale DLG's are sold in five categories: (1)

Public Land Survey System; (2) boundaries (3)

transportation; (4) hydrography; and (5) hypsography. All

DLG data distributed by the USGS are DLG - Level 3 (DLG-3),

which means the data contain a full range of attribute

codes, have full topological structuring, and have passed

certain quality-control checks.

Purpose:

DLG's depict information about geographic features on or near

the surface of the Earth, terrain, and political and administrative

units.  These data were collected as part of the National Mapping

Program.

Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date: 19870619

Ending_Date: Present

Currentness_Reference: publication date

Status:

Progress: In work

Maintenance_and_Update_Frequency: Irregular

Spatial_Domain:

Bounding_Coordinates:

West_Bounding_Coordinate: -124.7333

East_Bounding_Coordinate: -067.9500

North_Bounding_Coordinate: 49.3833

South_Bounding_Coordinate: 24.5333

Keywords:

Theme:

Theme_Keyword_Thesaurus: None.

Theme_Keyword: digital line graph

Theme_Keyword: DLG

Theme_Keyword: hydrography

Theme_Keyword: transportation

Theme_Keyword: boundaries

Theme_Keyword: U.S. Public Land Survey System

Theme_Keyword: hypsography

Place:

Place_Keyword_Thesaurus:

U.S. Department of Commerce, 1977, Countries,  dependencies,

areas of special sovereignty, and their principal administrative divisions

(Federal Information Processing Standard 10-3):Washington, D.C.,

National Institute of Standards and Technology.

Place_Keyword: US

Place_Keyword: CA

Place_Keyword: MX

Place_Keyword_Thesaurus:

U.S. Department of Commerce, 1987, Codes for the identification

of the States, the District of Columbia  and the outlying areas of

The United States, and associated areas

(Federal Information Processing Standard 5-2):Washington ,D. C.,

National Institute of Standards and Technology.

Place_Keyword: FIPS codes for states covered

Access_Constraints: None

Use_Constraints:

None.  Acknowledgement of the U.S. Geological Survey would

be appreciated in products derived from these data.

Data Quality Information

Data_Quality_Information:

Attribute_Accuracy:

Attribute_Accuracy_Report:

Refer to Positional_Accuracy element.

Logical_Consistency_Report:

Topological requirements include: lines must begin and end at

nodes, lines must connect to each other at nodes, lines do not

extend through nodes, left and right areas are defined for each

line element and are consistent throughout the file, and the

lines representing the limits of the file (neatline) are free of

gaps.

The tests of logical consistency were performed by the USGS

PROSYS program.  The neatline was generated by connecting the

four corners of the digital file, as established during initialization

of the digital file.  All data outside the enclosed region were ignored

and all data crossing these geographically straight lines were clipped at

the neatline. Data within a specified tolerance of the neatline

were snapped to the neatline.  Neatline straightening aligned the

digitized edges of the digital data with the generated neatline,

that is, with the longitude/latitude lines in geographic

coordinates.  All internal areas were tested for closure using

PROSYS.

Completeness_Report:

For Digital Revision Status = Not revised

Data completeness reflects the content of the source quadrangle.

Features may have been eliminated or generalized on the source

graphic, due to scale and legibility constraints.

For Digital Revision Status = Limited Update

This file has undergone limited update digital revision.  The

file contains only (1) those features that are photoidentifiable

on monoscopic source, supplemented with limited ancillary source,

and (2) those features, present on the original source quadrangle,

that can not be reliably photoidentified but that are not

considered particularly prone to change.

For Digital Revision Status = Standard Update

This file has undergone standard update digital revision. The

data completeness of this file meets NMD standards for feature

content.

Positional_Accuracy:

Horizontal_Positional_Accuracy:

Horizontal_Positional_Accuracy_Report: For Digital Revision Status = Not revised:

Vertical_Positional_Accuracy:

Vertical_Positional_Accuracy_Report:

For Hypsography and Hydrography Only:

For Digital Revision Status = Not revised:

Lineage:

Source_Information:

Source_Citation:

Citation_Information:

Originator:

U.S. Geological Survey or another mapping agency in

cooperation with USGS.

Publication_Date: Unknown

Title:

The Topographic Map Names Data Base designation for

the source quadrangle.

Geospatial_Data_Presentation_Form: map

Publication_Information:

Publication_Place: Reston, Virginia

Publisher: U.S. Geological Survey

Source_Scale_Denominator: 100000

Type_of_Source_Media: stable-base material

Source_Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date: 19870619

Ending_Date: Present

Source_Currentness_Reference: ground condition

Source_Citation_Abbreviation: MAP1

Source_Contribution: spatial and attribute information

Process_Step:

Process_Description:

For Digital Revision Status = Not digitally revised:

This Digital Line Graph was digitized from the USGS source

quadrangle, by either the National Mapping Division, one of

their cooperators, or one of their contractors.  The digital data

were produced by one of the following methods:

- scanning a stable-based copy of the graphic materials.

The scanning process captured the digital data at a

scanning resolution of at least 0.001 inches;  the

resulting raster data were vectorized and then attributed

on an interactive editing station.

- scanning the paper map.  The scanning process captured

the digital data at a scanning resolution of at least

0.001 inches;  the resulting raster data were vectorized

and then attributed on an interactive editing station.

- scanning a stable-based copy of the graphic materials.

The resulting raster data were then manually digitized

and attributed on an interactive editing station.  The

resolution of the digital data is at least 0.001 inches.

- scanning the paper map.  The resulting raster data were

then manually digitized and attributed on an interactive

editing station.  The resolution of the digital data is

at least 0.001 inches.

- manually digitizing from a stable-based copy of the

graphic material using a digitizing table to capture the

digital data at a resolution of at least 0.001 inches;

attribution was performed either as the data were

digitized, or on an interactive edit station after the

digitizing was completed.

- manually digitizing from the paper map using a digitizing

table to capture the digital data at a resolution of at

least 0.001 inches;  attribution was performed either as

the data were digitized, or on an interactive edit

station after the digitizing was completed.

The determination of the DLG production method was based on

various criteria, including feature density, feature

symbology, and availability of production systems.  Four

control points corresponding to the four corners of the

quadrangle were used for registration during data

collection.  An eight parameter projective transformation

was performed on the coordinates used in the data collection

and editing systems to register the digital data to the

internal coordinates used in PROSYS, and a four parameter linear

transformation was performed from the PROSYS internal coordinates

to Universal Transverse Mercator (UTM) grid coordinates.  The DLG

data were checked for position by one or more of the following

processes:

- comparing plots of the digital data to the graphic source.

- comparing the digital data to the digital raster scan.

DLG data classification was checked by at least one of the

following processes.

- comparing plots of the digital data to the graphic source

- comparing the digital data to the digital raster scan.

Source_Used_Citation_Abbreviation: MAP1

Process_Date: Unknown

Spatial Data Organization Information

Spatial_Data_Organization_Information:

Indirect_Spatial_Reference:

For Public Land Survey System:

U.S. Department of the Interior, Bureau of Land Management:

U.S. Public Land Survey System

For Boundaries:

U.S. Department of Commerce, 1977, Countries, dependencies, areas

of special sovereignty, and their principal administrative divisions.

(Federal Information Processing Standard 10-3): Washington,D.C.,

National Institute of Standards and Technology.

U.S. Department of Commerce, 1987, Codes for the identification

of the States, the District of Columbia  and the outlying areas of

The United States, and associated areas

(Federal Information Processing Standard 5-2): Washington, D. C.,

National Institute of Standards and Technology.

U.S. Department of Commerce,1990, Counties and equivalent

entities of The United States, its possessions, and associated areas

(Federal  Information Processing Standard 6-4):  Washington, D.C.

National Institute of Standards and Technology.

Direct_Spatial_Reference_Method: Vector

Spatial Reference Information

Spatial_Reference_Information:

Horizontal_Coordinate_System_Definition:

Planar:

Grid_Coordinate_System:

Grid_Coordinate_System_Name: Universal Transverse Mercator

Universal_Transverse_Mercator:

UTM_Zone_Number: 10-19

Transverse_Mercator:

Scale_Factor_at_Central_Meridian: .09996

Longitude_of_Central_Meridian: -123 00 00

Latitude_of_Projection_Origin: 0.0

False_Easting: 500000

False_Northing: 0.0

Planar_Coordinate_Information:

Planar_Coordinate_Encoding_Method: coordinate pair

Coordinate_Representation:

Abscissa_Resolution: 2.54

Ordinate_Resolution: 2.54

Planar_Distance_Units: meters

Geodetic_Model:

Horizontal_Datum_Name: North American Datum 1927

Ellipsoid_Name: Clark 1866

Semi-major_Axis: 6378206.4

Denominator_of_Flattening_Ratio: 294.98

Vertical_Coordinate_System_Definition:

Altitude_System_Definition:

Altitude_Datum_Name: National Geodetic Vertical Datum of 1929

Altitude_Resolution: 1

Altitude_Distance_Units: feet or meters

Altitude_Encoding_Method: attribute values

Depth_System_Definition:

Depth_Datum_Name: Mean lower low water

Depth_Resolution: 1

Depth_Distance_Units: meters or feet

Depth_Encoding_Method: attribute values

Entity and Attribute Information

Entity_and_Attribute_Information:

Overview_Description:

Entity_and_Attribute_Overview:

DLG3 attribute codes are used to describe the physical and cultural

characteristics of DLG node, line, and area elements. Attribute codes

are used to reduce redundant information, provide enough reference

information to support integration with larger data base, and describe

the relationships between cartographic elements. Each DLG element

has one or more attribute codes composed of a three digit major code

and a four-digit minor code. For example, with the 1: 24,000-scale DLG

data, the line attribute code 050 0412 has a major code (050), meaning

hydrography, with a minor code (0412) meaning stream.

Entity_and_Attribute_Detail_Citation:

U.S.Department of the Interior, U.S. Geological Survey, 1987

Digital Line Graphs from 1:100,000-Scale Maps--Data Users Guide 2;

Reston, Virginia

A hypertext extract is available at:

URL: http://edcwww.cr.usgs.gov/glis/hyper/guide/usgs_dlg

Softcopies in ASCII, WordPerfect, and PostScript format are available at:

URL: ftp://mapping.usgs.gov/pub/ti/DLG/

U.S.Department of the Interior, U.S.Geological Survey, 1990,

Standards for Digital Line Graphs; Reston, Virginia

Softcopies in ASCII, WordPerfect, and PostScript format are available at:

URL: ftp://mapping.usgs.gov/pub/ti/DLG/

Distribution Information

Distribution_Information:

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Earth Science Information Center, U.S.Geological Survey

Contact_Address:

Address_Type: mailing address

Address: 507 National Center

City: Reston

State_or_Province: Virginia

Postal_Code: 20192

Contact_Voice_Telephone: 1-888-ASK-USGS

Hours_of_Service: 0800-1600

Contact_Instructions:

In addition to the address above there are other ESIC offices

throughout the country.  A full list of these offices is at:

URL: http://mapping.usgs.gov/esic/esic_index.html

Resource_Description: 1:100,000-scale digital line graphs

Distribution_Liability:

Although these data have been processed successfully on a computer system at the

U.S. Geological Survey, no warranty expressed or implied is made by the USGS

regarding the utility of the data on any other system, nor shall the act of

distribution constitute any such warranty. The Geological Survey will warrant the

delivery of this product in computer-readable format, and will offer appropriate

adjustment of credit when the product is determined unreadable by correctly adjusted

computer input peripherals, or when the physical medium is delivered in damaged

condition.  Requests for adjustment of credit must be made within 90 day from the date

of this shipment from the ordering site.

Standard_Order_Process:

Digital_Form:

Digital_Transfer_Information:

Format_Name: SDTS

Format_Version_Date: 199207

Format_Specification: Topological Vector Profile

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: URL: http://edcwww.cr.usgs.gov/glis/hyper/guide/100kdlgfig/states.html

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: URL:ftp://edcftp.cr.usgs.gov/pub/data/DLG/100K/

Offline_Option:

Offline_Media: CD-ROM

Recording_Format: ISO 9660

Compatibility_Information:

This CD-ROM can be used with all computer operating

systems that support CD-ROM as a logical storage

device. All text files on the disc are in ASCII

format. Data files are in ASCII or binary format.

Digital_Form:

Digital_Transfer_Information:

Format_Name: DLG

Format_Version_Date: 198805

Format_Specification: Optional

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: URL: http://edcwww.cr.usgs.gov/glis/hyper/guide/100kdlgfig/states.html

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: URL: ftp://edcftp.cr.usgs.gov/pub/data/DLG/100K/

Offline_Option:

Offline_Media: CD-ROM

Recording_Format: ISO 9660

Compatibility_Information:

This CD-ROM can be used with all computer operating

systems that support CD-ROM as a logical storage

device. All text files on the disc are in ASCII

format. Data files are in ASCII or binary format.

Offline_Option:

Offline_Media: Cartridge

Recording_Capacity:

Recording_Density: 3480

Recording_Density_Units: megabytes

Recording_Format:

ASCII; available unlabelled or with ANSI-standard labels; available

block sizes are multiples of 80 characters ranging from 8000 to 32720 characters.

Offline_Option:

Offline_Media: Cassette

Recording_Capacity:

Recording_Density: 8-mm High (4.5Gb)

Recording_Density_Units: gigabytes

Recording_Format:

ASCII; available unlabelled or with ANSI-standard labels; available

block sizes are multiples of 80 characters ranging from 8000 to 32720 characters.

Offline_Option:

Offline_Media: Cassette

Recording_Capacity:

Recording_Density: 8-mm Low (2.3Gb)

Recording_Density_Units: gigabytes

Recording_Format:

ASCII; available unlabelled or with ANSI-standard labels; available

block sizes are multiples of 80 characters ranging from 8000 to 32720 characters.

Fees:

The online copy of the data set (when available electronically) may

be accessed without charge. For cartridge, cassette, and CD-ROM the

costs are available at:

URL: http://edcwww.cr.usgs.gov/glis/hyper/order_info/prices#100KDLG

Ordering_Instructions:

Intermediate-scale DLG data are sold in 30-minute units that correspond

to the east or west half of USGS 30- by 60-minute 1: 100,000-scale

topographic quadrangle maps. Each 30-minute unit is produced and

distributed as four 15- by 15-minute cells, except in high-density areas,

where the 15-minute cells may be divided into four 7.5-minute cells.

The transportation category of data includes major transportation systems

collected in three separate overlays labeled: (1) Roads and Trails, (2) Railroads,

and (3) Pipelines, Transmission Lines, and Miscellaneous Transportation Features.

Although collected separately, the three files are distributed as a single unit.

Orders for the transportation category for a given area will include all

three files.

Dataset searching and ordering capabilities are available through the

Global Land Information System (GLIS) at:

URL: http://edcwww.cr.usgs.gov/webglis

Metadata Reference Information

Metadata_Reference_Information:

Metadata_Date: 199001

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: U.S. Geological Survey

Contact_Address:

Address_Type: mailing address

Address: 508  National Center

City: Reston

State_or_Province: Virginia

Postal_Code: 20192

Contact_Voice_Telephone: 1 703 648 4543

Metadata_Standard_Name: Content Standards for Digital Geospatial Metadata

Metadata_Standard_Version: 19940608

Metadata_Security_Information:

Metadata_Security_Classification_System: None

Metadata_Security_Classification: Unclassified

Metadata_Security_Handling_Description: None

|| USGS NSDI Clearinghouse Home Page ||

URL:http://edcwww.cr.usgs.gov/nsdi/html/dlg100/dlg100

Maintainer: edcnsdi@edcwww.cr.usgs.gov 
Last modified: 29 June 1998