The USGS Digital Elevation Model (DEM) data files are digital representations of cartographic information in a raster form. DEMs consist of a sampled array of elevations for a number of ground positions at regularly spaced intervals. The U.S. Geological Survey (USGS) produces these digital cartographic/geographic data files as part of the National Mapping Program. The DEM data for 7.5-minute units correspond to the USGS 1:24,000 and 1:25,000 scale topographic quadrangle map series for all of the United States and its territories. Each 7.5-minute DEM is based on 30- by 30-meter data spacing with the Universal Transverse Mercator (UTM) projection. Each 7.5- by 7.5-minute block provides the same coverage as the standard USGS 7.5-minute map series.

Four processes have been used to generate DEM data for 7.5-minute units. The first three processes have been discontinued:

- The Gestalt Photo Mapper II (GPM2) is an automated photogrammetric system designed to produce orthophotos, digital terrain data, and contours in subunits known as patches.

- Manual profiling from photogrammetric stereomodels uses stereoplotters equipped with three-axis electronic digital profile recording modules for scanning stereomodels along successive terrain profiles.

- Elevations are recorded by stereomodel digitizing of contours, in which digital contours are acquired on stereoplotters equipped with three-axis digital recording modules.

- Interpolation from digital line graph (DLG) hypsographic and hydrographic data. All data now being generated use this process.

Data Characteristics

Each 7.5-minute unit of DEM coverage (based on the 7.5-minute quadrangle) consists of a regular array of elevations referenced horizontally in the UTM projection coordinate system. Elevation units are in meters or feet relative to National Geodetic Vertical Datum of 1929 (NGVD 29) in the continental U.S.

Spatial Resolution .

The 7.5-minute DEM data are stored as profiles in which the spacing of the elevations along and between each profile is 30 meters. The number of elevations in a profile will vary because of the variable angle between the quadrangle's geographic boundary (neatline) and the UTM coordinate system. DEM data of moderate to high-relief terrain or generated from maps with terrain contour intervals greater than 10 feet are generally recorded in meters (DEM data of low-relief terrain or generated from contour maps with intervals of 10 feet or less are recorded in feet).

DEM Accuracy

The method of determining DEM accuracy involves computation of the root-mean-square error (RMSE) for linearly interpolated elevations in the DEM and corresponding true elevations from the published maps. Test points are well distributed and representative of the terrain. Collection of test point data and comparison of the DEM to the quadrangle hypsography are conducted by USGS quality-control groups.

The vertical accuracy of 7.5-minute DEMs is equal to or better than 15 meters. A minimum of 28 test points per DEM is required (20 interior points and 8 edge points). The accuracy of the 7.5-minute DEM data, together with the data spacing, adequately support computer applications that analyze hypsographic features to a level of detail similar to manual interpretations of information as printed at map scales not larger than 1:24,000 scale.

SDTS Format

The Spatial Data Transfer Standard (SDTS) distribution format is designed as a mechanism for the transfer of spatial data between various computer systems. The SDTS format is designed to transfer data with complete content transfer (no loss of information). Data quality reports provide complete metadata and documentation of processing. The SDTS is a FIPS standard intended to work between various platforms and various data models. There is limited software available at this time, which can accommodate the SDTS-formatted data.