DEM from USGS SDTS format to ArcView:

- USGS site at http://edcwww.cr.usgs.gov/doc/edchome/ndcdb/ndcdb.html has DEMs.

- Uncompress with WINZIP (http://winzip.com/winzip/download.html) with “TAR smart CR/LF conversion” turned off. Extract to working folder using WINZIP.

- Get the SDTS DEM to ArcView Grid Conversion Utility at http://www.engineering.usu.edu/cee/faculty/dtarb/SDTSext.html instructions for how to install this are located at the mentioned site (http://www.engineering.usu.edu/dtarb).

- Open ArcView with the STDS grid import and spatial analyst extensions checked on.

- Open a view: from the file menu drag to “import data source” and select “USGS STDS DEM” in pull down menu. Name file in output grid dialog box.

- Open DEM in ArcView.

DLG from USGS SDTS format to ArcView:

- USGS site http://edcwww.cr.usgs.gov/doc/edchome/ndcdb/ndcdb.html has DLGs.

- Uncompress with WINZIP  (http://winzip.com/winzip/download.html) with “TAR smart CR/LF conversion” turned off. Extract to working folder using WINZIP.

- Run Sdts2dlg DOS program located at ftp://ftp.blm.gov/pub/gis/sdts/dlg/. Run this program from EXPLORER. Run this program four times for a 1:000,000 dlg coverage.

- Get ArcView DLG to Shape script “dlg20a” from ESRI at http://andes.esri.com/arcscripts/beginsearch.cfm?lang=Avenue.

- Paste and run script in ArcView for each of the four coverages.

- Open DLG coverages in ArcView.

In ArcView:

- Using 7.5 quad paper map, select lines from DLG that describe the glaciers and save as separate (line) shapefile. Name this coverage: demname_gla_outline.shp.

- Edit the shapefile selecting all lines that describe one glacier and UNION the lines into one feature (from EDIT menu). Do for each glacier.

- Use script “cvtplply” to change the polyline(s) to a polygon (script located at ESRI Avenue script site). Run script from “window.”  Name this coverage: demname_gla.shp.

- Use script “grid_cliptopoly” to clip the DEM with the glacier and glacier+buffer (script located at ESRI Avenue script site).

Structure:

For each 1:100,000 map the following will be in separate files. The map files will be the map name.

- Metadata word document.

- Coverage names.

- Creator and date.

- Data quality statement.

- Distribution information.

- Coordinate system description.

- DLG glacier line coverage.

- Shapefile of glaciers.

- DEM of glaciers.

ESRI scripts used:

- “dlg20a” changes a USGS DLG file to a AV shapefile.

- “cvtplply” changes a polyline to a polygon.

- “grid_cliptopoly” clips a shape from a grid (DEM).